Warehouse Operations Baseline

Locus Origin & Array Optimization Analysis

DRAFT — Internal Use Only

June 2025

# 1. Facility Overview

Mid-size retail/e-commerce fulfillment center operating two product categories across five pick zones. The warehouse operates on a 5.5-day schedule with standard two-shift coverage.

| Floor Area | 200,000 sq ft |
| --- | --- |
| Clear Height | 28–32 ft |
| Pick Zones | 5 zones (A–E, Zone A nearest Pack/Ship) |
| Active SKUs | 5,000 |
| Product Categories | Apparel & Footwear (2,000 SKUs) ¦ Health & Beauty (3,000 SKUs) |
| Shifts | 2 shifts × 8 hrs, Mon–Fri ¦ 1 shift Saturdays ¦ Closed Sundays |
| Slotting Age | ~18 months since last full re-slot |
| Pick Methodology | Batch wave picking, 4–6 orders per wave, full-facility travel |

# 2. Throughput Baseline

## 2.1  Volume

| Weekday Orders / Day | 8,000 |
| --- | --- |
| Saturday Orders / Day | 4,000 |
| Weekly Orders | 44,000 |
| Annualised Orders | ~2,300,000 |
| Avg Lines / Order | 2.69 |
| Weekday Picks / Day | 21,508 |

## 2.2  Capacity Utilisation

Theoretical picker capacity: 16 pickers × 2 shifts × 8 hrs × 75 UPH = 19,200 picks/day.

| Picker Capacity (theoretical) | 19,200 picks / day |
| --- | --- |
| Actual Weekday Picks | 21,508 picks / day |
| Capacity Utilisation | 112% ← over capacity today |
| Daily Pick Deficit | 2,308 picks / day covered by overtime |

The facility cannot fulfill current weekday volume within a standard two-shift schedule at 75 UPH.

      Overtime is already required today — before any volume growth is factored in.

# 3. Picker Labor Model

| Gross Time per Pick | 48 sec |
| --- | --- |
| Active Pick Time | 20 sec (scan, locate, pick, place in tote) |
| Travel Time per Pick | 12.5 sec |
| Travel as % of Shift (pure walking) | 26% — travel alone |
| Walk Speed (assumed) | 3 mph (264 ft / min) |
| Travel Distance per Pick | ~55 ft per pick |
| Admin / Overhead per Pick | 15.5 sec (wave mgmt, system, minor delays — 32% of shift) |
| Walk Distance per Shift | ~6 miles per picker (industry standard: 5–8 miles) |
| Total Picker-Miles / Day | ~200 picker-miles (all pickers, both shifts) |

58% of every paid picker shift is not touching product — travel (26%) + admin/overhead (32%) combined.

      Origin attacks both: zone restriction eliminates travel; LocusONE + directed picking cuts overhead. That is the 25–35% UPH improvement.

# 4. Walk Distance Analysis

Zone distribution of picks — 60-day OB transaction data (1,032,284 picks):

| Zone | Dist. from Pack/Ship | Pick Share | Picks (60-day) |
| --- | --- | --- | --- |
| A | 50 ft | 45.3% | 467,205 |
| B | 150 ft | 28.6% | 294,953 |
| C | 275 ft | 15.0% | 155,017 |
| D | 400 ft | 8.1% | 84,001 |
| E | 525 ft | 3.0% | 31,108 |

| Weighted Avg Distance (current slotting) | 155 ft per pick |
| --- | --- |
| Weighted Avg Distance (ideal slotting) | 135 ft per pick |
| Excess Travel from Slotting Drift | 20 ft per pick (13% improvement potential from re-slot alone) |

# 5. Slotting Health

| SKUs with Tier Drift | 1,618 of 5,000 (32%) |
| --- | --- |
| A-tier SKUs in Zones D/E | 83 SKUs — 26,677 A-tier picks from far zones (3.2% of A-tier volume) |
| Waste Miles (A-tier misslots) | 30.8 picker-miles / day |
| C-tier SKUs in Zones A/B | 273 SKUs occupying prime slots needed by fast movers |
| Prime Ergonomic Picks (L2–L3) | 60.7% of picks at optimal height (18–54") |
| Suboptimal Ergonomic Picks | 9.0% of picks at floor or overhead height |
| Surge SKUs still in far zones | Athletic & Activewear (+2.5×), Skincare (+2.2×), Wellness (+2.2×) |

# 6. Labor Cost Baseline

| Role | Count / Shift | Rate | Daily Cost (2 shifts) |
| --- | --- | --- | --- |
| Pickers | 16 | $20 / hr | $5,120 |
| Receivers | 4 | $21 / hr | $1,344 |
| Replenishment | 5 | $21 / hr | $1,680 |
| Pack-Out | 6 | $19 / hr | $1,824 |
| Supervisors | 2 | $27 / hr | $864 |
| TOTAL | 33 | — | $10,832 |

| Cost per Order (all labor) | $1.35 |
| --- | --- |
| Cost per Order (picking only) | $0.64 |
| Cost per Unit Picked | $0.24 |
| Annual Labor Spend | ~$2,816,000 |
| Annual Overtime Cost (est.) | ~$240,000 (to cover 112% capacity gap) |

# 7. Optimization Analysis

## 7.1  Re-Slot Only  (no robots, recommended as prerequisite)

Correcting the 1,618 SKUs that have drifted since the last slot improves pick efficiency and sets up a clean zone structure for Origin deployment.

| UPH Improvement | 75 → 83 (+11%) |
| --- | --- |
| Travel Time After Re-Slot | ~24% (vs. 26% today — modest improvement) |
| Additional Picks / Day | +2,021 |
| Additional Orders / Day | +752 (capacity headroom restored) |
| Limitation | Travel + overhead still ~76% of shift. Drift will recur without ongoing slotting governance. |

## 7.2  Locus Origin Deployment

Origin is deployed after the re-slot is complete. Pickers become zone-restricted (1–2 zones each) and no longer travel to Pack/Ship — Origin handles tote transport autonomously via LocusONE orchestration.

How Origin changes the pick process:

1.  Eliminates zone transit travel  —  pickers stay in their zone for the full shift. No more walking 525 ft to Zone E and back.

      2.  Eliminates Pack/Ship trips  —  Origin carries completed totes to Pack/Ship autonomously.

      3.  Directed pick instructions  —  Origin's screen replaces RF gun + clipboard, reducing scan time and cognitive overhead.

      4.  Smarter wave management  —  LocusONE clusters picks tightly by zone, reducing within-zone travel.

Fleet sizing:  14 robots per shift, sized to match pickers at target throughput (1 robot per picker).

UPH improvement range (industry benchmark):

| Improvement | UPH | Picks / Day | vs. 21,508 Demand | Pickers / Shift |
| --- | --- | --- | --- | --- |
| Conservative (25%) | 94 | 24,064 | +12% headroom | 14 |
| Midpoint (30%) | 98 | 25,088 | +17% headroom | 14 |
| Optimistic (35%) | 101 | 25,856 | +20% headroom | 14 |

## 7.3  Scenario Comparison

| Metric | Current | Origin Only | Origin + Array |
| --- | --- | --- | --- |
| OB Pickers / shift | 16 | 7 | 2 |
| H&B pickers | 16 | ~10 | 0 (Array) |
| Apparel pickers | 16 | ~4 | 2 |
| Origin robots | 0 | 4 | 2 |
| Array robots | 0 | 0 | 4 |
| Receivers | 4 | 4 | 2 |
| Replenishment | 5 | 5 | 1 |
| Supervisors | 2 | 1 | 1 |
| Total headcount | 33 | 30 | 14 |
| Pick capacity / day | 19,200 | 21,952 | 21,760 |
| Capacity utilization | 112% (OT req) | 98% | 99% |
| Picker walk dist / shift | 6 mi | 4 mi | 2 mi |
| Annual robot cost | — | $72K (Origin) | $228K (Origin + Array) |
| Annual labor savings vs today | — | $359K + OT elim. | $1,628K + OT elim. |
| Net annual benefit | — | $287K | $1,400K |

Origin alone eliminates overtime and recovers capacity headroom for $287K net annual benefit.

      Origin + Array removes 72% of pick volume from human labor entirely, cuts headcount by 19, and delivers a 6x larger return — $1.4M net annual benefit before capital investment.

      H&B (aisles 1-2) and Apparel (aisles 3-4) are already physically segregated — Array deploys into H&B aisles with no cross-aisle reslot required.

# 7.4  Locus Array Deployment

Array is an autonomous R2G (robot-to-goods) system equipped with NeuraGrasp for unattended picking. At this facility Array is deployed in two roles: outbound H&B picking and inbound putaway / replenishment.

## 7.4.1  Outbound — H&B Picking

H&B represents 72.2% of daily pick volume (15,523 picks/day on weekdays) and is already physically segregated in aisles 1–2. Array operates those aisles continuously on a 20–24 hr/day schedule, removing all human pickers from H&B zones.

| H&B daily pick demand | 15,523 picks / day (72% of facility volume) |
| --- | --- |
| Array operating hours | 22 hrs / day (extended vs. 16-hr human shift) |
| Array robots deployed | 4 robots |
| Required rate per robot | ~176 picks / hr (mid-range for aisle-based R2G) |
| H&B pick capacity (4 robots) | 15,488 picks / day (100% coverage) |
| SKU separation | H&B (aisles 1-2) fully Array-operated ¦ Apparel (aisles 3-4) human + Origin |
| NeuraGrasp fit | H&B items (uniform boxes, bottles, packets) well-suited to robotic grasping |

Array pricing note:  $4,000 / robot / month is used in this model — reflecting Array's greater capability vs. Origin.

      This is a planning assumption. Actual rate requires confirmation from Locus account team.

## 7.4.2  Inbound — Putaway and Replenishment

Current inbound operations: 89,164 units/day received across 157 lines. Two-step flow — dock to floor-stacked reserve (Zones D/E) then forward pick face. Today this requires 4 receivers and 5 dedicated replenishment workers, with IB/OB traffic conflicts throughout both shifts.

| Units received / day | 89,164 (avg) |
| --- | --- |
| Receipt lines / day | 157 |
| Replenishment case moves / day | ~4,211 (all categories) |
| Current IB labor | 4 receivers + 5 replenishment = 9 workers |
| Array IB role | Autonomous putaway (dock to reserve) and case replenishment (reserve to pick face) |
| Array IB operating window | Overnight and gap shifts — no IB/OB conflict |
| Labor impact | Receivers: 4 → 2 ¦ Replenishment: 5 → 1 exception handler |
| Traffic conflict resolution | Array replenishment runs off-peak; Zones D/E vacated during OB shift |

## 7.4.3  H&B Reslot Assessment

The warehouse is already category-segregated — no cross-aisle reslot is required for Array deployment. The H&B slotting picture:

| Item | Finding | Action |
| --- | --- | --- |
| Category segregation | H&B in aisles 1-2, Apparel in aisles 3-4 — already separated | None — deploy Array into existing aisles |
| Ergonomic tiers | Human golden zone (L2-L3) no longer applies — Array reaches all 5 levels equally | Remove ergonomic constraint from H&B slotting rules |
| Fast-mover drift | 79 H&B SKUs (2.6%) are top-20% velocity but located in Zones D/E | One-time 79-location move during Array commissioning |
| Ongoing governance | LocusONE task data enables continuous velocity-based slot optimization | Built into LocusONE — no manual reslot projects |

The reslot for Array is 79 SKU moves — not a project, just part of commissioning.

      Array actually improves H&B density: all 5 rack levels become usable, whereas today only L2-L3 are ergonomically ideal for humans.

## 7.5  Capital Investment Requirements

Array requires two categories of capital investment: (1) handling infrastructure at the pick interfaces, and (2) racking compatibility in the H&B aisles. Three racking scenarios are modeled below.

## Infrastructure  (required in all scenarios)

| Item | Description | Est. Cost |
| --- | --- | --- |
| Pack/Ship drop-off station | Robot deposit point at Pack/Ship: short conveyor segment or tote-transfer chute. Array deposits filled totes; pack-out team processes downstream. | $25,000 – $35,000 |
| Receiving pickup station | Staging handoff point at inbound dock. Array picks up cases from a consistent height and position for autonomous putaway. Includes floor markers and safety perimeter. | $15,000 – $25,000 |
| Infrastructure subtotal | Common to all racking scenarios | ~$50,000 |

## Racking Scenarios  (H&B aisles 1-2)

H&B racking footprint: 3,000 bin locations, ~1,000 rack bays, ~2,000 linear ft across aisles 1-2.

| Scenario | Racking Scope | Rack Cost | + Infra | Total CapEx | Payback* |
| --- | --- | --- | --- | --- | --- |
| A — Compatible | Existing racking confirmed compatible with Array arm reach and aisle width via site survey. No structural rack work required. | $0 | + $50K | ~$50K | < 1 month |
| B — Retooling | Adjust beam heights, widen select aisles, re-anchor uprights. Partial scope driven by site survey findings. | ~$150K | + $50K | ~$200K | ~1.7 months |
| C — Full Replacement | Replace all H&B racking with Array-specified shelving. Full aisle rebuild — purpose-built for robot throughput and density. Unlocks all 5 levels for usable inventory. | ~$350K | + $50K | ~$400K | ~3.4 months |

* Payback calculated against $1,400K net annual OpEx benefit (Section 8.4).  Full rack replacement pays back in under 4 months.

      Recommended first step: commission a site survey to determine which scenario applies before budgeting CapEx.

      Racking CapEx is customer-owned.  Array RaaS subscription covers robots, LocusONE licensing, and ongoing maintenance.

# 8. ROI Model

## 8.1  What Is Included in This Calculation  (Origin — Phase 1)

The following savings are calculated directly from facility data and stated labor rates:

| Savings Item | Basis | Annual Value |
| --- | --- | --- |
| Overtime elimination | 2,308 daily pick deficit × 30.8 OT hrs/day × $30/hr OT rate × 260 days | $240,240 |
| Picker headcount reduction | 16 → 14 pickers/shift (weekdays) + 16 → 13 (Saturdays) × rates | $191,360 |
| Total Included Labor Savings | — | $431,600 |

## 8.2  Robot Fleet Cost  (Origin — Phase 1)

| Origin robots | 4 robots (Apparel aisles 3-4) |
| --- | --- |
| Rate Used | $1,500 / robot / month |
| Annual Fleet Cost | $72,000 (4 × $1,500 × 12) |
| Break-Even Rate | $2,571 / robot / month (below this = positive ROI) |

## 8.3  Net ROI Summary  (Origin — Phase 1)

| Annual Labor Savings | $431,600 |
| --- | --- |
| Annual Fleet Cost | $72,000 |
| Net Annual Savings | $359,600 |
| Annual ROI | 499% on fleet cost |
| CapEx Required | None (RaaS model — pure OpEx) |
| Cash Flow | Positive from month one |

Phase 1 (Origin): $360K net annual benefit, zero CapEx, positive cash flow from day one.

      Origin operates independently and delivers full ROI without Array — Phase 2 is additive.

## 8.4  Array ROI  (Phase 2  —  Origin + Array Combined)

Adding Array to the Origin deployment eliminates human labor from H&B entirely and extends inbound operations with overnight autonomous putaway and replenishment.

## 8.4.1  Labor Savings vs. Current State

| Savings Item | Basis | Annual Value |
| --- | --- | --- |
| Picker reduction (OB) | 16 → 4 pickers/shift x $20/hr x 8 hrs x 2 shifts x 260 days | $998,400 |
| Receiver reduction (IB) | 4 → 2 receivers x $21/hr x 8 hrs x 1 shift x 260 days | $87,360 |
| Replenishment reduction | 5 → 1 x $21/hr x 8 hrs x 2 shifts x 260 days | $349,440 |
| Supervisor reduction | 2 → 1 x $27/hr x 8 hrs x 2 shifts x 260 days | $112,320 |
| Overtime elimination | 2,308 daily pick deficit fully eliminated | $80,011 |
| Total Labor Savings | — | $1,627,531 |

## 8.4.2  Robot Fleet Cost  (RaaS)

| Origin robots (Phase 1 carry-over) | 2 robots (Apparel aisles only) |
| --- | --- |
| Origin rate | $1,500 / robot / month |
| Origin annual cost | $36,000 |
| Array robots | 4 robots (H&B aisles + IB putaway/replenishment) |
| Array rate | $4,000 / robot / month (planning assumption — confirm with Locus account team) |
| Array annual cost | $192,000 |
| Total annual fleet cost | $228,000 |

## 8.4.3  Net ROI — Origin + Array  (OpEx)

| Annual Labor Savings | $1,627,531 |
| --- | --- |
| Annual Fleet Cost | $228,000 |
| Net Annual Benefit (OpEx) | $1,399,531 |
| vs. Origin-Only | $360K (Phase 1) → $1,400K (Phase 2) — 3.9x improvement |
| CapEx Required | See Section 7.5 (site-dependent, ~$50K – ~$400K) |

## 8.4.4  CapEx Payback Summary

| Racking Scenario | Total CapEx | Payback at $1,400K/yr net benefit |
| --- | --- | --- |
| A — Existing rack compatible | ~$50K | < 1 month |
| B — Rack retooling | ~$200K | ~1.7 months |
| C — Full rack replacement | ~$400K | ~3.4 months |

At $4,000/robot/month for Array, the fleet cost is $228K/yr against $1.6M in labor savings — a 7:1 return on robot spend.

      Even worst-case CapEx ($400K full rack replacement) is recovered in under 4 months of operation.

      Array pricing is a planning assumption. Final rate requires confirmation from the Locus account team prior to proposal.

## 8.5  Items Not Included — For Collaboration with Customer Financial Team

The following value drivers apply to both Origin and Origin + Array deployments. They are real but require customer-specific data to quantify accurately, and should be explored jointly with the customer's finance and operations leadership:

| Value Driver | Description | Data Needed from Customer |
| --- | --- | --- |
| Throughput expansion revenue | Same headcount enables +17% order capacity with Origin. Array + extended hours push this further. Revenue enablement value can be significant if absorbing new channels or peak seasons. | Revenue per order, peak volume forecasts, cost of turning away orders |
| Mispick error reduction | Directed pick instructions typically reduce error rates from ~0.5% to <0.1%. At 21,500 picks/day, ~85 fewer errors daily. | Current error/mispick rate, cost per error (return shipping + reshipment + CS) |
| Worker safety & ergonomics | Reduced cart pushing and fatigue lowers musculoskeletal injury risk. Array removes humans from H&B entirely. | Historical injury rates, workers comp premiums, turnover cost per picker |
| Slotting governance value | LocusONE data enables continuous slotting optimization, preventing drift that created today's 32% out-of-position rate. | Cost of periodic re-slot projects today |
| IB dock-to-stock time | Array autonomous overnight putaway compresses dock-to-stock time. Value depends on whether faster availability reduces stockouts or improves fill rate. | Current dock-to-stock SLA, stockout frequency, lost sales estimate |

Note:  The ROI models in Sections 8.3 and 8.4 are intentionally conservative — using only directly calculable labor line items.

      Any of the items above would increase net benefit materially.

      Recommended next step: joint financial workshop with customer operations and finance teams.

# 9. Recommendations

| Phase | Action | Expected Outcome | Net Annual Benefit |
| --- | --- | --- | --- |
| Pre-deploy — Immediate | Re-slot the 83 A-tier SKUs in Zones D/E and 273 C-tier SKUs in Zones A/B | Restore zone structure, gain +11% UPH, clean zone assignment for robot deployment | No robot spend — labor efficiency gain only |
| Phase 1 — Origin | Deploy 4 Locus Origin robots in Apparel aisles (3-4) with LocusONE orchestration. Re-slot first, then deploy. | Eliminate OT, reach ~98 UPH for Apparel, reduce picker headcount. Positive cash flow from day one. | $360K / yr (no CapEx) |
| Phase 2 — Array | Commission site survey to confirm racking scenario. Deploy 4 Array robots in H&B aisles (1-2) for OB picking and overnight IB putaway/replenishment. | Remove H&B picking from human labor entirely. Overnight autonomous replenishment eliminates IB/OB aisle conflict. | $1,400K / yr ($50K-$400K CapEx — < 4 month payback) |
| Ongoing — All phases | Use LocusONE pick data to govern slotting continuously — review velocity tiers monthly | Prevent slotting drift recurrence; maintain optimized zone structure as demand evolves | Preserves Phase 1 and Phase 2 ROI |

Two-phase deployment de-risks the program: Origin delivers positive ROI independently in Phase 1.

      Phase 2 (Array) is additive — the Origin infrastructure, LocusONE integration, and slotting work from Phase 1 reduce Array deployment effort.

      Combined steady-state: 14 headcount (down from 33), $1.4M net annual benefit, 99% capacity utilization.